Daffodil International University

Department of Electrical and Electronic Engineering (EEE)

COMPLEX ENGINEERING ASSIGNMENT/PROJECT REPORT COVER PAGE

Project Title or Problem Statement: DUAL-LM393 TEMPERATURE CONTROLLER & SAFETY CLAMP

| Course Title: Electronics Laboratory | Course Code: 0714-222 |
| --- | --- |

Student ID(s): 251-33-096

251-33-097

251-33-099

| Instructor's Name: Engr. Bijoy Sorker |  | Submission Date: 16-08-2026 |
| --- | --- | --- |
| Batch: 251 | Semester: Summer-2026 | Level-Term: 2-2 |

Program: B.Sc. in Electrical and Electronic Engineering

Assessment Mapping (Instructor Use Only)

| Task Statement | CO No. | Mapped PO | WK | WP/EA | Bloom’s Taxonomy Domain Level | Assigned Marks | Obtained Marks | Total (30) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Design and formulate a novel idea for an electronic system as a team using basic components and ICs (555 Timer and/or Op- Amp) | CO1 | PO5 |  | P1, P3 | C6 | 5 |  |  |
| Analyze the system mathematically and simulate it to verify its operation under real-world constraints. |  |  |  | P2, P3 | C4 | 10 |  |  |
| Build and implement the hardware, troubleshooting practical circuit issues to demonstrate the system working according to the intended objective. |  |  |  | P2, P3 | C6 | 12 |  |  |
| Prepare and evaluate a technical report and a presentation in a team to explain your system, results, and collective contribution. | CO2 | PO9 |  |  | C5 | 13 |  |  |

Signature of the Instructor

| Course Code: | 0 | 7 | 1 | 4 | - | 2 | 2 | 2 | Course Title: | Electronics Laboratory |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Addressing the Range of Complex Engineering Problem Solving (WP)

| Attribute (WP) | Mapped | Please write your comments against each attribute mapped in the space below. Attach more sheets if necessary. (Optional) |
| --- | --- | --- |
| P1: Depth of knowledge required | √ | The project requires students to understand and apply knowledge of basic electronic components, ICs, sensors, and microcontrollers. They must integrate theory with practical application, which ensures a deep understanding of electronics concepts and their real-life usage. |
| P2: Range of conflicting requirements | √ | Students must manage wide-ranging conflicting design considerations and real-world technical issues while building the hardware prototype. They are required to balance competing factors such as component compatibility, cost-efficiency, power stability, and physical wiring constraints to ensure the system works effectively according to its intended objective. |
| P3: Depth of analysis required | √ | Students must mathematically analyze their system, perform simulations, and compare results with expected performance. This involves critical thinking, problem-solving, and technical analysis, fulfilling the requirement for thorough analytical depth. |
| P4: Familiarity of issues |  |  |
| P5: Extent of applicable codes |  |  |
| P6: Extent of stakeholder involvement and conflicting requirements |  |  |
| P7: Interdependence |  |  |

# Electronics Laboratory – Complex Engineering Project Evaluation Rubric (Maximum Marks: 40)

| Task | Excellent (90– 100%) | Good (70–89%) | Satisfactory (50–69%) | Needs Improvement (<50%) | Marks |
| --- | --- | --- | --- | --- | --- |
| Idea/Concept & Novelty | Idea is highly innovative, clearly defined, and well-justified | Idea is moderately innovative and feasible | Idea is basic, limited novelty | Idea is unclear or not feasible | 2 |
| Circuit/System Design | Complete, well-structured design integrating components, ICs; clear schematic provided | Mostly complete design with minor missing details; schematic mostly clear | Design is partially complete or missing key connections | Design incomplete or incorrect; schematic missing | 3 |
| Mathematical Analysis & Simulation | Thorough analysis, correct calculations, clear simulation results fully explained | Moderate analysis, minor errors, simulation results mostly correct | Basic analysis, some errors, limited simulation results | Analysis missing or incorrect, simulation not done | 10 |
| Hardware Implementation & Functionality | Hardware built successfully; system works fully as intended | Hardware mostly works; minor issues in functionality | Hardware partially works; several functional issues | Hardware not built or does not work | 12 |
| Report & Presentation | Well-structured, clear, complete report; presentation is professional and informative | Report mostly complete with minor gaps; presentation understandable | Report incomplete; presentation unclear | Report missing or very poor; no presentation slide | 13 |

DUAL-LM393 TEMPERATURE CONTROLLER
& SAFETY CLAMP

Project Report

# 1. Abstract

This project presents a three-stage temperature controller and safety-clamp system based on two LM393 dual comparator ICs, a TMP36 temperature-sensing characteristic, an NE555 low-frequency oscillator, and an NPN transistor switching stage. A 10 kΩ potentiometer is used as a simulator for the temperature-dependent input voltage. The system defines three temperature levels: 30 °C for Stage 1 warning, 60 °C for active alarm and cooling, and 100 °C for an independent dual master shutdown clamp. The corresponding sensor voltages are 0.80 V, 1.10 V, and 1.50 V. Between 60 °C and 100 °C, the fan is activated and the red LED and buzzer are driven by an approximately 1.5 Hz NE555 oscillator. At or above 100 °C, the clamp comparators force the stage-control nodes LOW.

# 2. Introduction

## 2.1 Background

Temperature monitoring and protection are important in electronic systems because excessive heat can affect normal operation and may damage components. This project uses different temperature levels to provide a clear response as the temperature increases. The system first gives a warning, then activates cooling and an alarm, and finally provides a master shutdown at a higher temperature. The design combines temperature-voltage conversion, LM393 comparator threshold detection, LED indication, NE555 timing, and transistor switching in one practical control system.

## 2.2 Motivation

The main motivation of this project is to develop a simple and practical temperature protection system using commonly available electronic components. It demonstrates how a changing temperature signal can be converted into voltage and compared with selected reference levels to control different outputs. The project also demonstrates warning indication, an audible alarm, automatic fan cooling, and a final shutdown function. These features make the project useful for understanding basic temperature monitoring and safety-control circuits.

## 2.3 Objectives

1. Simulate the temperature input with a 10 kΩ potentiometer over 0–5 V.

2. Generate a Stage 1 warning at 30 °C (0.80 V).

3. Activate the alarm and cooling stage at 60 °C (1.10 V).

4. Provide independent master shutdown at 100 °C (1.50 V).

5. Generate an approximately 1.5 Hz flashing/beeping output with the NE555.

6. Check LED, comparator, and transistor currents using the stated component values.

# 3. Methodology

## 3.1 Block Diagram

[10 kΩ Temperature Simulator / V_IN]
              ↓
[LM393 IC1 — Stage 1 & Stage 2 Comparators]
       ↓                         ↓
 Yellow LED                N_STAGE2
                              ↓
                ┌─────────────┴─────────────┐
                ↓                           ↓
          [2N2222 Fan Driver]          [NE555 Astable]
                ↓                       ↙       ↘
             5 V Fan              Red LED     Piezo Buzzer

[LM393 IC2 — Dual 100 °C Clamp] → Master Shutdown

## 3.2 Circuit Diagram

Figure 1. Complete circuit diagram of the Dual-LM393 Temperature Controller & Safety Clamp.

Figure 2. Breadboard implementation / simulation view of the temperature-controlled fan and alarm circuit.

Figure 3. PCB implementation / simulation view of the temperature-controlled fan and alarm circuit.

## 3.3 Working Principle

The input voltage follows the specified TMP36 transfer function: V_OUT = 0.50 V + (10 mV/°C × T). The 10 kΩ potentiometer reproduces this voltage for testing. IC1 compares V_IN with 0.80 V and 1.10 V references. IC2 compares V_IN with the 1.50 V reference using two independent clamp channels. The Stage 2 node controls the NE555 RESET input and the NPN transistor base through a 1 kΩ resistor.

## 3.4 Components List

| Component | Quantity | Purpose |
| --- | --- | --- |
| 9 V battery | 1 | Power source |
| LM7805 voltage regulator | 1 | 5 V regulated supply |
| LM393 dual comparator ICs | 2 | Temperature threshold detection |
| NE555 timer IC | 1 | Alarm oscillator |
| 10 kΩ potentiometers | 4 | Reference/sensor-voltage adjustment |
| 4.7 kΩ resistor | 1 | Circuit bias/current limiting |
| 1 kΩ resistors | 2 | Timing/current limiting |
| 10 kΩ resistors | 1 | Pull-up/bias networks |
| 47 kΩ resistor | 1 | NE555 timing network |
| 330 Ω resistors | 2 | LED current limiting |
| Capacitors | 5 | Supply decoupling and timing |
| Diodes | 1 | Protection/flyback |
| NPN transistor | 1 | Fan/motor switching |
| DC motor/fan | 1 | Cooling output |
| Buzzer | 1 | Audible alarm |
| LED indicators | 2 | Status indication |
| Breadboards | 2 | Circuit assembly |
| Jumper wires | As required | Circuit connections |

## 3.5 Operating Truth Table

| V_IN | Temperature | N_STAGE1 | N_STAGE2 | Yellow LED | Red LED | Buzzer | Fan | State |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| < 0.80 V | < 30 °C | 0 V | 0 V | OFF | OFF | OFF | OFF | System Idle |
| 0.80–1.10 V | 30–60 °C | +5 V | 0 V | ON | OFF | OFF | OFF | Stage 1 Warning |
| 1.10–1.50 V | 60–100 °C | +5 V | +5 V | OFF | BLINKING | PULSING | ON | Stage 2 Active Alarm & Cooling |
| ≥ 1.50 V | ≥ 100 °C | 0 V | 0 V | OFF | OFF | OFF | OFF | Over-Temp Master Shutdown |

# 4. Results & Discussion

## 4.1 Calculation Results

The calculated results support the selected operating thresholds and alarm timing. Using VOUT = 0.50 + 0.010T, the reference voltages are 0.80 V at 30 °C, 1.10 V at 60 °C, and 1.50 V at 100 °C.

For the NE555 astable stage, RA = 1 kΩ, RB = 47 kΩ and CT = 10 µF give f = 1.44/[(RA + 2RB)CT] ≈ 1.52 Hz, so the alarm output is approximately 1.5 Hz.

Therefore, the expected result is: below 30 °C the system remains idle; from 30–60 °C the Stage 1 warning operates; from 60–100 °C the fan and alarm operate; and at or above 100 °C the master shutdown condition is activated.

### 4.1.1 Temperature-to-Voltage Mapping (TMP36 Transfer Function)

• Formula: V_OUT = 0.50 V + (10 mV/°C × Temperature in °C)

• Stage 1 Trigger (30 °C): V_REF1 = 0.50 V + (0.010 V/°C × 30 °C) = 0.80 V

• Stage 2 Trigger (60 °C): V_REF2 = 0.50 V + (0.010 V/°C × 60 °C) = 1.10 V

• Master Shutdown Clamp (100 °C): V_REF3 = 0.50 V + (0.010 V/°C × 100 °C) = 1.50 V

### 4.1.2 NE555 Low-Frequency Astable Oscillator Calculations

• Component Values: R_A = 1 kΩ, R_B = 47 kΩ, C_T = 10 µF (0.00001 F)

• High Output Time (t_HIGH): 0.693 × (R_A + R_B) × C_T = 0.693 × 48,000 Ω × 0.00001 F = 0.3326 s

• Low Output Time (t_LOW): 0.693 × R_B × C_T = 0.693 × 47,000 Ω × 0.00001 F = 0.3257 s

• Total Period (T): t_HIGH + t_LOW = 0.3326 s + 0.3257 s = 0.6583 s

• Oscillation Frequency (f): 1.44 / [(R_A + 2 × R_B) × C_T] = 1.44 / [95,000 × 0.00001] = 1.5158 Hz (~1.5 Hz)

• Duty Cycle (D): [ (R_A + R_B) / (R_A + 2 × R_B) ] × 100% = (48 / 95) × 100% = 50.53%

## 4.2 Simulation

The simulation result shows the temperature-control circuit operating with reference and sensor voltages set for the selected thresholds. The simulation display shows 0.80 V, 1.10 V and 1.50 V levels, with a 5.00 V supply.

Figure 4. Simulation result of the temperature-controlled fan and alarm circuit.

## 4.3 Experimental

The experimental circuit was assembled on breadboards and tested using a digital multimeter. The photograph shows the physical implementation and a measured voltage of approximately 1.487 V.

Figure 5. Experimental setup and measured result of the temperature-controlled fan and alarm circuit.

# 5. Conclusion

The Dual-LM393 Temperature Controller and Safety Clamp successfully demonstrates three-stage temperature monitoring and protection. The system provides a warning at 30 °C, activates the alarm and cooling fan at 60 °C, and provides master shutdown at 100 °C. The NE555 generates approximately 1.5 Hz alarm signal, while the LM393 comparators control the different temperature stages. The experimental measurement of approximately 1.487 V was also close to the 1.50 V shutdown reference. Overall, the project demonstrates a practical combination of temperature threshold detection, alarm indication, and automatic cooling control.

# 6. References

1. Texas Instruments. LM393 Dual Differential Comparator — product information and datasheet. https://www.ti.com/product/LM393

2. Texas Instruments. NE555 Precision Timers — product information and datasheet. https://www.ti.com/product/NE555

3. Analog Devices. TMP36 Voltage Output Temperature Sensor — product information and datasheet. https://www.analog.com/en/products/tmp36.html

4. onsemi. 2N2222A Small Signal Switching Transistor — datasheet. https://www.onsemi.com/download/data-sheet/pdf/2n2222a-d.pdf